Impact of Environmental Regulations on Carbon Emissions of

Transportation Infrastructure: China’s Evidence

Jingxiao Zhang1, Weixing Jin2, Simon P Philbin3, Qingchang Lu4, Pablo Ballesteros-Pérez5, Martin Skitmore6, Hui Li7

- Professor, School of Economics and Management, Chang’an University, Shaanxi Xian, P.R .China, 710064, zhangjingxiao964@126.com

- Lecturer, School of Civil Engineering, Chang’an University, Shaanxi Xi’an,

P.R. China, 710061. Email: jinweixing1993@126.com

- Professor, Nathu Puri Institute for Engineering and Enterprise, London South Bank University, London, United Kingdom, philbins@lsbu.ac.uk

- Professor, School of Electronic and Control Engineering, Chang’an University, Shaanxi Xian, P.R. China, 710064, qclu@chd.edu.cn

- Senior researcher, Departamento de Ingeniería Mecánica y Diseño industrial, Escuela       Superior       de       Ingeniería,        Universidad        de        Cádiz,   Spain, pablo.ballesteros@uca.es

- Professor, School of Civil Engineering and Built Environment, Queensland University of Technology (QUT), Brisbane, Australia, rm.skitmore@qut.edu.au

- Associate Professor, School of Civil Engineering, Chang’an University, Shaanxi Xian, P.R. China, 710061, lihui9922@chd.edu.cn

Corresponding Author:

Jingxiao Zhang, Professor, School of Economics and Management, Chang’ an University, Xian, Shaanxi, People's Republic of China, 710061, Email: zhangjingxiao964@126.com

Declarations of interest: none.

- Impact of Environmental Regulations on the Carbon Emissions

- of Transportation Infrastructure: China’s Evidence

- Abstract:  Environmental  regulations  (ER)  can support  or dissuade the reduction of

- carbon emissions. However, there are limited studies on the impact of ER on carbon

- emissions  for  the  transportation  infrastructure.  Using  provincial  panel  data   from

- China for the period 2001-2017, this  study measures  how ER  can affect  the  carbon

- emissions  efficiency (CEE) of transportation  infrastructure.  The study uses Hansen’s

- panel threshold model to analyze current thresholds of ER on CEE. The results show

- that (1) China's transportation infrastructure during 2001-2017 experienced a 10.15%

- decrease of CEE; (2) there are significant heterogeneities across the regions of China

- regarding  CEE;  (3)  ER  produces  some  CEE  threshold  effects  in  China's current

- transportation infrastructure. This study proposes alternative evaluation methods and

- theoretical   frameworks   for   dynamically   measuring   the   CEE   of transportation

- infrastructure, and provides governments with improved criteria to design effective

- environmental policies.

16

- Keywords: Transportation infrastructure; Carbon emission efficiency; Environmental

- regulations; Low-carbon transition; Threshold model.

# 1 Introduction

- According to the World Resources Institute (WRI), global greenhouse gas (GHG)

- emissions increased from 31.78 billion (109) tons in 1990 to 47.57 billion tons in 2017

- (WRI, 2017); of these GHG, CO2 emissions account for approximately 75%. As a

- developing but responsible country, China has committed to limit CO2 emissions and

- reach  the peak CO2 emissions as early as  2030 (Zhou  et al., 2019a).  In this  context,

- various industries are facing greater pressure to reduce CO2 emissions. In China the

- transportation is a major emitter of waste CO2. Indeed, the CO2 emissions for the

- transportation infrastructure in China increased at an average annual rate of 15%

- during the  period  1997-2006  (Bai  et  al.).  Hence,  there  is  an  urgent  need  for the

- transportation infrastructure sector in China to undergo a low-carbon transition if the

- country wants to meet its CO2 emissions reduction objectives.

- Due to the existence of environmental externalities and market failures, the

- government needs to adopt environmental regulations (ER) to solve the increasingly

- serious  environmental   pollution  problems  (Zhang  et   al.,  2020).  However,  some

- scholars question the necessity for ER. For example, Schou (2002) argues that the

- (negative) externality of environmental pollution will always remain under the control

- of market mechanisms,  dismissing ER  as superfluous.  In  a similar vein, Sinn (2008)

- believes that implementation of ER may actually lead to a further increase in CO2

- emissions through decreasing CO2 emission efficiency. Therefore, to avoid potentially

- opposite effects, it is important to have an improved understanding of how to measure

- the cause and effect of ER on transportation infrastructure.

- It has been postulated that increasing the carbon emissions efficiency (CEE) is

- the key to achieve a low-carbon transportation infrastructure sector (Cao and Li,

- 2014).  However,  the literature  concerning evaluation  of CEE  has  mostly adopted a

- static  perspective  with  less  attention  paid  to  the  dynamic  efficiency  of  carbon

- emissions,  especially  for  transportation  infrastructure.  Similarly,  studies  of  the

- impacts of ER on CEE are mainly concentrated in the industrial sector (Ouyang et al.,

- 2020), manufacturing industry (Zhai and An, 2020), construction industry (Zhang et

- al., 2018), and energy industry (Du et al., 2020). The impact of ER on the CEE of

- transportation   infrastructure   remains   relatively   unexplored.   Only   by accurately

- assessing  the  carbon  emissions  efficiency  of  transportation  infrastructure  and the

- corresponding  impact  of  environmental  regulations  can  development  policies  be

- formulated to achieve a low-carbon transformation of transportation infrastructure.

- In response to the problems above, this research study adopts transportation

- infrastructure as the research object. Based on the panel data of 30 Chinese provinces

- and  cities  from  2001  to  2017,  the  study evaluates  the  dynamic  carbon emissions

- efficiency of transportation infrastructure according to both the national and regional

- levels, which  allows exploration of the threshold  effect of environmental regulations

- on the carbon emission efficiency of transportation infrastructure.

- The main contributions of this paper are as follows: (1) According to the

- dynamic  perspective  to  measure  the  carbon  emissions  efficiency of transportation

- infrastructure,   this   study   applies   a   RAM-based   global   Malmquist-Luenberger

- productivity  index   (GML)   to  dynamically  evaluate  the   CEE   of   transportation

- infrastructure.  The  study  will  improve  current  evaluation  methods  and theoretical

- frameworks to measure CEE in transportation infrastructure. (2) The study analyzes

- the threshold effects of ER on CEE of Chinese transportation infrastructure, thereby

- clarifying the impact of environmental regulations on carbon emission efficiency that

- will help to strengthen the role of ER in promoting low-carbon transformation. (3)

- The study supports improvement in the effectiveness of ER implementation, and is

- therefore  of  great  significance  for  improving  a  country's  governance  system  and

- capabilities in the field of ecological civilization for critical infrastructure.

- The rest of the article is organized as follows: Section 2 presents the literature

- review; Section 3 provides the research methods and data; Section 4 provides the

- empirical  results;  Section  5  is  the  discussions  and  policy  recommendations;  and

- Section 6 finishes with the conclusion and proposed further research.

# 2 Literature review

## 2.1 Carbon emission efficiency measurement

- Carbon emissions efficiency refers to the efficiency of production activities that

- generate a corresponding output while inducing carbon dioxide emissions. Carbon

- emissions  efficiency (CEE) is  generally divided  into two types:  static and dynamic.

- Compared to static CEE, dynamic CEE reflects changes of efficiency over time. This

- provides  a  clearer  reference  for  formulating  improved  low-carbon  transformation

- policies.

- In recent years, Carbon emissions efficiency (CEE) has attracted significant

- attention in the research literature. However, a single unified definition of CEE does

- not yet exist. Mielnik and Goldemberg (1999) applied the ratio of CO2 emissions to

- energy consumption to represent CEE, but named it the carbon index. The carbon

- index  is now  an  important  measure  of  a  country's  energy efficiency (Ang, 1999).

- However, both carbon productivity and the carbon index are measured by the ratio of

- CO2 emissions to a single variable, ignoring the effect of the many other confounding

- variables, such as the economic development level, industrial structure, and factor

- substitution (Zhou et al., 2010).

- In order to compensate for this oversimplification, some studies have proposed a

- multi-dimensional concept of carbon emissions efficiency, that is, when calculating

- the  production  technology  efficiency,  the  undesired  output  of  CO2  emissions  is

- included in the evaluation index system. That is why in this research study, we will

- consider  CEE   within   a   comprehensive  multi-dimensional   index,   referring  to a

- technical efficiency that takes CO2 emissions into account.

- Regarding CEE in the transportation industry, most studies have focused on

- comparative performance analyses of different types of transportation or geographical

- regions. Chang et al. (2013) analyzed the provincial CEE of China’s transportation

- system in 2009 using a slacks-based measure (SBM) model. Wang and He (2017)

- calculated the marginal abatement cost of CO2 emissions and analyzed the CEE of

- China’s   regional  transportation  sectors  in  the   2007–2012  period.  They  found   a

- negative correlation between CEE and its marginal abatement cost.

- On the other hand, it can be observed that Data Envelopment Analysis (DEA)

- has become the mainstream method for carbon emissions efficiency research. Zofı́O

- and Prieto (2001) used the DEA method to perform a comparative analysis of OECD

- countries based on their respective carbon emission efficiencies. Choi et al. (2012)

- estimated carbon emissions reduction costs with a SBM model. Gómez-Calvet et al.

- (2014)  also  applied  the  slacks-based  measure  (SBM)  method  to  investigate  and

- measure the CEE of EU (European Union) countries. To reduce the subjectivity and

- deviations of SBM, Sueyoshi et al. (2010) extended the study of Cooper et  al. (2000),

- and proposed  a Range  Adjusted Measure model that considered  undesirable outputs,

- which avoids setting any parameters subjectively. As a result, this approach improved

- the objectivity and accuracy of CEE measurement.

- Oh (2010) proposed a Global Malmquist-Luenberger productivity index, which

- solved  the  problem  of  infeasible  solutions.  Later,  Emrouznejad  and  Yang  (2016)

- combined a Range Adjusted Measures model with the Global Malmquist-Luenberger

- productivity index.  The new  RAM-based  global Malmquist-Luenberger productivity

- index handles undesirable variable outputs and slacks, while is also able to capture

- dynamic changes in CEE. Since these developments, the index has been considered as

- a reliable method for measuring dynamic CEE.

## 2.2 Influencing factors of carbon emissions efficiency

- Recently, more scholars have carried out research on the factors affecting carbon

- emission efficiency from varied perspectives. Most studies have considered factors

- such  as  population,  economy,  industrial  structure,  and  energy structure. Wang  and

- Wang  (2020)  studied  the  influencing  factors  of  short-term  and  long-term  carbon

- emissions and found that energy efficiency and trade openness are the main factors

- affecting carbon emissions. They did	not only studied the impact of urbanization on

- carbon  emissions efficiency,  but  also discussed the impact of economic development

- levels,   industrial   structure,   energy   consumption   structure,   foreign   trade   and

- government intervention. Furthermore, Sheng et al. (2020) studied the long-term and

- short-term effects of economic growth on carbon emissions efficiency, and found that

- economic  growth  can  improve  carbon  emissions  reduction  efficiency.  Shen  et al.

- (2019) examined the impact of industrial development levels, foreign trade levels and

- geographic location on carbon emission efficiency.

- There are also scholars conducting research on different types of traffic and

- industries. The study of  Wang  et al. (2011) concluded that the main factors  affecting

- the  CEE  of  China’s  cargo  transportation  sector  were  the  regional  economic  and

- industrial development level, energy utilization efficiency,  as well as the  number and

- size of cargo transportation enterprises. Zhou et al. (2019b) found that technological

- progress  and  energy structure  can  improve  the  carbon  emissions  efficiency of the

- construction industry, and the impacts are different in different industries. Moreover,

- environmental regulations have gradually become one of the important influencing

- factors in scholars' research. For example, Pan et al. (2019) examined the impact of

- China's environmental regulations on carbon emissions efficiency. Pei et al. (2019)

- used a threshold regression model to test the nonlinear relationship between

environmental regulations and carbon emissions.

## 2.3 Impact of environmental regulations on carbon emission efficiency

- Environmental regulations (ER) are policies adopted by the government to

- reduce   environmental   externalities,   with   the   goal   of   enabling   environmental

- protection and sustainable economic development. Porter (1991) was among the first

- to propose that  appropriate ER  could promote technological innovation and improve

- resource utilization – resulting in the eventual improvement of technical efficiency.

- This is called the Porter  Hypothesis.  In order to  test the Porter Hypothesis,  scholars

- have conducted several empirical studies based on different assumptions, population

- samples, analysis methods, and selection of different variables (Lanjouw and Mody,

- 1996, Jaffe and Palmer, 1997, Gray, 1987).

- However, current research on the Porter effect can be classified as possessing

- one of two opposing views. Jaffe and Palmer (1997) as well as Brunnermeier and

- Cohen (2003) support Porter's hypothesis and  believe that environmental  regulations

- have a positive impact on industrial technical efficiency; Whereas Gray (1987) and

- others support the opposite. Hence, despite more research being developed, there still

- is  no  definitive  answer.  Lanoie  et  al.  (2008)  analyzed  the  impact  of  ER  on  the

- productivity   of   the   manufacturing   industry   in   Canada,   finding   short-term vs.

- long-term  inconsistencies.  Alpay  et  al.  (2002)  analyzed  the  impact  of  ER  on the

- productivity of the food industry in the United States and Mexico during the period

- 1971-1994,  and  found  clear  differences  between  the  two  countries  in  regard  to

- adopting the same ER. Wang et al. (2019) used a threshold model to analyze the

- nonlinear relationship between environmental regulations and carbon  emissions.  The

- study investigated the case of 30 provinces in China and found that ER exhibits a

- threshold effect on CO2 emissions that is accompanied with significant differences

- among the eastern, central, and western regions of the country.

- Regarding the impact of ER on CEE, Pei et al. (2019) examined the provincial

- panel data of energy-intensive industries for the period 2005-2015 in order to assess

- the impact of ER on CEE, finding that ER can directly improve CEE. However, they

- also found that ER can also indirectly improve the CEE by improving the technical

- efficiency of the industry.  Furthermore,  Zhao et  al. (2015) found that different  types

- of environmental regulations have different effects on efficiency and carbon emissions.

- In   this  case,   command   and   control  as   well   as government  subsidies  improve

- efficiency and reduce carbon dioxide, whereas market incentives appear to have little

- influence. More recently, Zhang et al. (2018) adopted a three-stage DEA method to

- evaluate  the  CEE  of  the  construction  industry  in  China,  finding  that  there  is  a

- threshold effect of ER.

## 2.4 Knowledge gap

- From the developed literature review, there are three limitations in previous

- studies:

- (1) The current research on carbon emission efficiency mostly focuses on the

- static  perspective,   and   many  of  the   existing  studies   ignore  the  differences   in

- production frontiers in different periods. There are relatively few dynamic studies on

- carbon emissions efficiency.

- (2) Carbon emissions efficiency with multi-dimensional characteristics has now

- become the focus of research, but most of the existing studies focus on the automotive

- industry as  well  as  the  impact  of  factors  such  as  population,  economy, industrial

- structure,   and   energy   structure.   Few   studies   have   focused   on   the   effect  of

- environmental	regulations	on	carbon	emissions	efficiency	of	transportation

- infrastructure.

- (3) There are currently different (opposing) opinions on the impact of

- environmental  regulations  on  carbon  emissions  efficiency.  Some  empirical studies

- that are based on a single perspective are single-sided, and it is therefore necessary to

- analyze the impact of environmental regulations on carbon emissions efficiency from

- multiple perspectives (such as the threshold effect, etc.).

# 3 Research methods

## 3.1 RAM-based global Malmquist-Luenberger productivity index (GML)

- GML can effectively deal with undesired output and slack variables, while

- avoiding the use of subjective model parameters. This numerical approach improves

- the accuracy and  comparability of  efficiency measurement results. Hence, this  study

- combines the RAM model with the global Malmquist-Luenberger production index to

- construct  a  RAM-based  global  Malmquist-Luenberger  productivity  index (GML).

- This enhanced model will be used to measure the dynamic efficiency of China’s

- transportation infrastructure carbon emissions from 2001 to 2017, thereby accurately

- reflecting  the  variation  of  carbon  emissions  efficiency  in  the  17-year  period  of

- analysis.

- In this study, the transportation infrastructure of each Chinese province is

- supposed as a Decision-Making Unit (DMU) and is represented as DMUj (j = 1,2,3  ...

- n) assuming that in time period (t = 1,2,3 ... T) each DMU requires m inputs and

- produces s desirable outputs and k undesirable outputs. These are respectively noted

m×n

s×n

219	as	I = (i1, i2, … , im) ∈ R+	,	O = (o1, o2, … , os) ∈ R+	and

k×n

UO = (uo1, uo2, … , uok) ∈ R+	.   Based   on   these   assumptions,   the   production

- function is:

Pt = {(Ot, UOt): I tcan produce(Ot, UOt)}

I ∈ R+	，O ∈ R+ , ，UO ∈ R+

(1)

m×n

s×n

k×n

- However, this study aims to measure the CEE of transportation infrastructure

- from the perspective of total factor productivity (TFP). To overcome the problem of

- aggregation of inputs and outputs, a DEA model is therefore needed.  As shown in the

- literature review, a RAM model does not involve subjectivity when setting the model

- parameters (Wang and Yuan, 2020). Hence, following the approach of (Sueyoshi et al.,

- 2010), a RAM model considering undesirable output can be constructed as follows:

min0 = 1 − (RtTdt + RtTdt  + +RtT dt  )	(2)

I	I	0    0	u0   u0

∑n	}.jlt + dt  = lt

j=1	j	I	O

∑n	}.jOt − dt  = Ot

s. t.

j=1	j	0	O

∑n	}.jUOt + dt	= UOt

j=1	j	u0	O

l∑n

}.j = 1, dt, dt , dt , }.j ≥ 0

where:

j=1

I	0	u0

RtT = (R1t, R2t, … , Rmt)T

I	I	I	I

RtT = (R1t, R2t, … , Rst)T

0	0	0	0

229	RtT   = (R1t , R2t  , … , Rkt )T	(3)

u0	u0	u0	u0

230	In equation (2), }.j is the combination coefficient of each input and output

231

variable of a DMUj. When }.j satisfies the condition ∑n

}.j = 1, the RAM model is

j=1

∑

232	under the assumption of variable returns to scale (VRS). When the constraint of

233

n j=1

}.j = 1 is removed, the RAM model turns into being under the assumption of

- constant  returns  to  scale  (CRS).  dt ,  dt  and  dt	represent the slack of input,

I	0	u0

- desirable output,  and  undesired  output  respectively,  and  reflect the degree of input

- redundancy  and  output  shortage.  Meanwhile,  0  is  static  CEE  measured  from the

- perspective of TFP. This variable always satisfies 0 ≤ 1.

- However, evaluating CEE solely with a RAM model may have the following

- problems: (a) the calculation would be very complex; (b) it would be impossible to

- analyze the spatial and temporal changes of CEE. To overcome these problems, this

- study combined  the RAM model with   a  global  Malmquist-Luenberger productivity

- index (GML) as shown below:

→G	(It+1, Ot+1, UOt+1) MLPG(It, Ot, UOt, It+1, Ot+1, UOt+1) = D DDF

→G	(It, Ot, UOt)

D DDF

→G	(Ip, Op, UOp) = minθ = 1 − (RpTdp + RpTdp + RpT dp )

D DDF

I	I	0	0

u0 u0

∑T      ∑n    AjtIt+dp=Ip

t=l    j=l	j

∑T     ∑n     Ajt0t-dp =0p

s. t.

t=l    j=l	j	0

(4)

∑T     ∑n     Ajtu0t+dp   =u0p

t=l

j=l

j	u0

Ajt�O,j=1,..,n;t=1,..,T

- where:

RtT = (R1t, R2t, … , Rmt)T

I	I	I	I

RtT = (R1t, R2t, … , Rst)T

0	0	0	0

245	RtT   = (R1t , R2t  , … , Rkt )T	(5)

u0	u0	u0	u0

246

In equation 4, MLPG represents dynamic CEE.

→G	is the Directional

D DDF,V

247

Distance Function (DDF) under the assumption of VRS, and

→G	represents the

D DDF,C

- DDF under the assumption of CRS. In particular, under the assumption of VRS,

- MLPG can be decomposed as:

V

MLPG(It, Ot, UOt, It+1, Ot+1, UOt+1)

t

PTEt+1

BPGt,t+1	SEt+1(It+1, Ot+1, UOt+1)

= PTEt

× [ 	t+1 ] × ( BPGt,t+1

SEt(It, Ot, UOt)	) =

=  PECt,t+1  × BPCt,t+1 × SCHt,t+1	(6)

→G	(It,0t,u0t) t( t	t)	DDDF,C

SE    I , O	=

→G	(It,0t,u0t)

DDDF,V

where PTEt represents Pure Technical Efficiency at time t, while PECt,t+1

t

represents the change of PTE between time t and t + 1. Additionally, BPGt，t+1

(7)

- represents the Best Practice Gap between the contemporaneous technology frontier

- and global technology frontier. Accordingly, to measure the technological changes

- between the t and t  + 1  periods, we use  BPCt，t+1    to denote the change  in  BPG.

- Meanwhile,  SCHt,t+1   denotes  the  ratio  of  two  scale  efficiencies  for  the  two

- periods under the global benchmark technology.

## 3.2 Hansen’s panel threshold model

- Hansen’s panel threshold model is introduced to analyze the threshold effect.

- This model can identify the limit values of independent variables that can cause a

- turning point in the dependent variable. These thresholds, of course, depend on the

- data characteristics and reveal the influence of independent variables on the explained

- variable  by  partition  (Ji  et  al.,  2019).  Therefore,  Hansen’s  panel  threshold model

- avoids  the  subjectivity  and  arbitrariness  of  artificial  classification,  improving the

- accuracy  of  the  regression  model  results.  The  general  form  of  Hansen’s  panel

- threshold model is:

- yit = ui + xitf31 ∙ l(qit ≤ y) + xitf32 ∙ l(qit > y) + 0X + Eit	(9)

- where i  represents the DMU and t represents the year, whereas f31, f32, and 0  are

- corresponding  estimated  coefficients.  yit,  xit,  and  qit   are  the  explained  variable,

- independent  variables  and  threshold  variable  respectively.  Additionally,  y   is  the

- threshold value, X is a set of control variables, and  Eit  is the random error.  l(∙)  is

- the  index   function,  and   l(∙)=1,  when   the  condition  in  parentheses  is  satisfied,

- otherwise l(∙)=0. The panel data is partitioned according to the threshold variable qit

- and the threshold value γ.

- However, equation (9) represents a single threshold model. Based on this, a

- double threshold model and a triple threshold model can also be built as follows:

yit = ui + xitf31 ∙ l(qit ≤ y1) + xitf32 ∙ l(y1 < qit ≤ y2)

278	+xitf33 ∙ l(qit  > y2) + 0x + Eit	(10)

yit = ui + xitf31 ∙ l(qit ≤ y1) + xitf32 ∙ l(y1 < qit ≤ y2)

279	+xitf33 ∙ l(y2 < qit ≤ y3)+xitf34 ∙ l(qit > y3) + 0x + Eit	(11)

- respectively. Here, the single threshold, double threshold, and triple threshold models

- are applied progressively to test the potential threshold effects.

## 3.3 Data

- This study selects panel data from 2001 to 2017 in 30 provinces of China, except

- Hong Kong, Macau, Taiwan and Tibet because the data from these regions was not

- readily available. The panel data used in this study come from China’s Statistical

- Yearbook (2001-2018) and China’s Energy Statistical Yearbook (2001-2018).

## 3.3.1 Carbon emissions efficiency evaluation indicator system

- (1) Input indicators

- This research study uses an index system supported in the existing research

- literature on index selection. For further details of the specific index system employed,

- see  Table  A-3  in  Appendix  A.  The  inputs  of  transportation  infrastructure mostly

- comprise labor, capital, and energy. In general, labor input is measured by laboring

- hours. However, due to the lack of relevant statistical data, we used the number of

- engaged persons in the transportation industry to represent labor input. Based on the

- investment  in  fixed  assets,  transportation  industry  capital  was  calculated  by  the

- perpetual inventory method. Details of the calculations are shown in Appendix B of

- the   Supplemental   Material.   As   for   the   energy   input,   eight   primary   energy

- consumptions  were  selected:  raw  coal,  coke,  gasoline,  kerosene,  diesel,  fuel  oil,

- liquified petroleum gas, and natural gas. These were then converted to their standard

coal equivalent as shown in the Table A-1 of Appendix A.

- (2) Desirable output indicators

- In general, desirable outputs of transportation infrastructure mainly comprise two

- components, namely economic value and practical value. This study used the added

- value of  the  industry to  represent  economic  value. Meanwhile, practical value  was

- represented by cargo turnover and passenger turnover as in the study of (Liu et al.,

2017).

- (3) Undesirable output indicators

- As mentioned earlier, we evaluated CEE from the perspective of TFP. Therefore,

- the undesirable output of transportation infrastructure is represented by CO2 emissions.

- The  carbon  emissions  coefficient  method  was  adopted  for  the  calculation  as

- recommended  by the  United  Nations  Intergovernmental  Panel  on  Climate Change

- (IPCC) (IPCC, 2006). The specific equation is as follows:

CO2 = ∑n

Ei × NCVi × CEFi × COFi × (44)	(12)

i=1

12

- where Ei and NCVi represent the terminal consumption and net calorific value of the

- ith energy respectively, CEFi and COFi represent the carbon emissions factor and

- carbon oxidation ratio, 44/12 corresponds to the coefficient of carbon conversion, and

- the CO2 emissions of each energy source can be calculated as in Table A-2 of

Appendix A.

## 3.3.2 Explained variable

- The explained variable is the CEE of the transportation infrastructure as

- measured by a RAM-based global Malmquist-Luenberger productivity index (GML).

As shown earlier, this index is noted as GML.

## 3.3.3 Independent variable and threshold variable

- ER is the independent variable in the Hansen’s panel threshold model and ER is

- also the threshold variable adopted in this approach. The content of ER is complex,

- and not only includes the environmental laws and policies implemented nationwide,

- but also some regional or industrial regulations.  All of these  regulations  may affect

- CEE. In order to fully reflect the role of ER, by referring to previous studies, the

- ratio of the total input of environmental governance to the GDP of each province was

- selected. This characterizes the intensity of ER from an overall perspective (Zhang et

al., 2018).

## 3.3.4 Control variables

- The control variables comprise the regional economic development level,

- technology innovation level, institutional factor, and opening-up level.

- Previous	studies	identified	a	relationship	between	regional	economic

- development level and regional industry development (Zheng and Wang,  2019b).  In

- general,  high  economic  development  levels  can  lead  to  upgrades  of  industrial

- structure, thereby optimizing resources allocation and increasing industry TFP (Zhou

- et al., 2019c). The natural logarithm of per capita GDP of each province was selected

- to  represent  the  regional  economic  development  level,  which  is  expressed  as

- LnECO.

- Improvement of the technology innovation level helps to improve the

- companies’ resource utilization efficiency and reduce environmental pollution (Luo

- et al., 2019). The technology innovation level may also have an impact on CEE.

- (Quazi and Talukder, 2011) found a significant positive correlation between  regional

- technology  innovation  level  and  its  educational  level.  Therefore,  regional higher

- education  proportion  of  each  province  was  selected  to  represent  its  technology

- innovation level – hereinafter denoted as RES.

- Previous studies also found that the regional opening-up level has a significant

- impact on the transportation industry’s  TFP (Chu et  al., 2014).  On the one hand,  an

- environment  conducive   for  foreign  direct  investment   (FDI)   would  promote  the

- introduction of technology and human resources. This could contribute to improving

- technical  efficiency.  On  the  other  hand,  transportation  infrastructure  is  highly

- dependent  on  investment.  Hence,  the  introduction  of  foreign  investment may also

- potentially cause an increase of CO2 emissions, leading to a decrease in CEE. Here,

- the natural logarithm of regional FDI is used to represent the opening-up level,

- expressed by LnOPEN.

- The institutional factor is regarded as an important factor influencing industrial

- development. Currently, China has encouraged the participation of social capital in

- transportation infrastructure construction, despite the government still being the major

- investor (Ren et al., 2011). In general,  greater regional government investment means

- lower  regional  marketization.  This  may  lead  to  a  lack  of  motivation  for  the

- introduction of new technologies, materials, and energy. Of course, not all of these are

- conducive to reducing carbon emissions. Hence, the ratio of state-owned assets to the

- whole society's specified asset investment in the transportation industry, expressed as

- OW,   was  selected  to  represent  the  institutional  factor.  The  descriptive  statistical

- results of all the above variables are shown in Table 1.

- Table 1. The descriptive statistical of all variables.

|  | Variables | Sample size | Min | Max | Mean | SD |
| --- | --- | --- | --- | --- | --- | --- |
|  | CEE | 480 | 0.779 | 1.201 | 0.993 | 0.031 |
|  | ER | 480 | 0.3 | 4.24 | 1.325 | 0.672 |
|  | LnECO | 480 | 8.056 | 11.768 | 10.156 | 0.774 |
|  | RES | 480 | 0.018 | 0.476 | 0.01 | 0.066 |
|  | OW | 480 | 0.101 | 0.658 | 0.323 | 0.117 |
|  | LnOPEN | 480 | 7.31 | 15.096 | 12.21 | 1.779 |
| 374 |  |  |  |  |  |  |

# 4 Results

## 4.1 Data test for regression analysis

- In order to ensure the accuracy of the regression analysis, panel unit root tests

- and panel co-integration tests were used to characterize the panel data. Details of

these two methods are provided in Appendix C of the Supplemental Material.

- (1) Panel unit root test

- The Augmented Dickey-Fuller (ADF) test was used to determine whether the

- panel data of each variable had a unit root. Then, the Phillips-Perron (PP) test and

- Im–Pesaran–Shin (IPS) test were used to verify the results. As shown in Table 1, the

- results of the three methods are highly consistent. This means the null hypothesis of

- the  existence  of  unit  root  is  strongly  rejected,  indicating  that  all  variables  are

stationary.

- Table 2. Results of the panel unit root test.

|  | ADF test | PP test | IPS test |
| --- | --- | --- | --- |
| Variable | Z statistic | Z statistic | W statistic |
|  | (p value) | (p value) | (p value) |
| GML | -4.62753*** (0.00060) | -14.53050*** (0.0000) | -4.18153*** (0.0000) |
| ER | -1.69712** | -3.23160*** | -1.55785* |

(0.0448)	(0.0006)	(0.0596)

Ln ECO

-5.79856*** (0.0000)

-8.09060*** (0.0000)

-5.53524*** (0.0000)

RES

-7.17679*** (0.0000)

-2.45952*** (0.0070)

-6.89297*** (0.0000)

-1.67012**

OW

(0.0474)

-4.92068*** (0.0000)

-1.82117** (0.0343)

Ln OPEN

-5.23641*** (0.0000)

-3.46886*** (0.0003)

-7.62693*** (0.0000)

- Note: ***, **, and * represent statistically significant at the 1%, 5%, and 10% levels

- respectively.

- (2) Panel co-integration test

- After the panel unit root test, the Kao panel co-integration test was used to

- investigate the co-integration relationship of each variable. As shown in Table 2, the

- T  statistic  value  was  -1.406589  and  the  p  value  0.0000.  This  means  there  is a

- long-term co-integration relationship between the variables, and that regression  tests

can be directly performed on the panel data.

- Table 3. Panel co-integration test results.

ADF

T statistic	p value

-12.43575***	0.0000

| Residuals | 0.002481 | - |
| --- | --- | --- |
| HAC variance | 0.000575 | - |

- Note: ***indicates statistically significant at the 1% level.

## 4.2 Dynamic carbon emissions efficiency analysis

- Analysis of the carbon emissions efficiency of the transportation infrastructure

- was carried out at two levels, namely the national level and the regional level. At the

- national level, the data of all 30 provinces are used for analysis; at the regional level,

- the 30 provinces are divided into three regions: east, middle and west, and the carbon

- emissions efficiency in the three regions is analyzed in each of them.

## 4.2.1 National level

- For further details on the dynamic efficiency and decomposition of carbon

- emissions from transportation infrastructure at the national level go to Table A-4 in

- Appendix A.

- From 2001 to 2017, the CEE of the transportation infrastructure showed a

- declining-rising-declining  trend.  Based  on  this  trend,  the  observation  period was

- further divided into three stages as follows: 2001-2006, 2007-2012, and 2013-2017.

- These periods are represented in different figures  later.  Due to the long  observation

- period, if the GML and its decomposition are accumulated and rebased on the year

- 2001, it would be difficult to observe variations in each stage. Therefore, we set the

- starting year of each of the three stages as the base year for comparison.

- As shown in Figure 1, in time period 2001-2016, GML decreased year by year

- with an average annual decline of 1% and a cumulative decline of approximately

- 4.93%. From the decomposition of the GML, the average annual decline rates of

- PEC (changes in technological efficiency), BPC (technological change), and SCH

- (change in scale efficiency) were 0.06%, 0.83%, and 0.41% respectively. This is due

- to   the   low   level   of   technological   innovation   in   the   field   of transportation

- infrastructure  during  the  period.  It  may  also  have  to  do  with  an  insufficient

- utilization of new energy sources, which could not effectively promote technological

Efficiency value

progress.

1.020

1.000

1.000

1.001

0.998

0.995

0.993

1.004

0.998

1.003

0.993

1.001

0.997

0.988

0.980

0.976

0.976

0.980

0.960

0.940

GML PEC BPC SCH

2001

0.977

0.975

0.957

0.959

0.951

2002

2003

Year

2004

0.953

2005

2006

Figure 1. GML and its decomposition during 2001-2006.

- As shown in Figure 2, during 2007-2012, GML showed an overall growth trend,

- at an average annual rate of 0.24% and a cumulative increase of approximately

- 1.43%. This indicates that the CEE improved slightly. From the decomposition of

- GML, the average annual growth rates of PEC and BPC were 0.03% and 0.13%

- respectively, which promoted the growth of CEE in this period. The reason for this

- change may have been that technology advancements in transportation infrastructure

- led to small productivity improvements. However, during this period, SCH still had

- an overall declining trend (average annual decline of 0.64% and cumulative decrease

- of about  3.60%).  The deterioration  of economies  of  scale is  the main  reason   for

- limiting the further growth of carbon emissions efficiency during this period, and

Efficiency value

resource allocation still needs to be improved.

1.040

1.020

1.019

1.000

1.009

1.001

1.006

1.005

1.000

0.993

0.999

0.989

1.006

1.003

0.992

1.014

1.008

1.001

0.980

0.997

0.985

0.968

1.003

0.992

0.960

GML

PEC

BPC SCH

0.988

0.982

0.961

0.964

0.947

0.940

2006

2007

2008

2009

Year

2010

2011

2012

Figure 2. GML and its decomposition during 2007-2012.

- As shown in Figure 3, GML sharply declined during 2013-2017, with an

- average annual  decrease  rate of 1.48%. This  highlights  that  the carbon   emissions

- efficiency of China's transportation infrastructure has dropped significantly. From the

- decomposition results, BPC and SCH also decreased significantly (average annual

- declination rates of 1.6% and 1.14% respectively). The significant decline in BPC

- means that the technological frontier in the same period is approaching the global

- technological frontier. This along the direction of more carbon dioxide emissions and

- less  expected  output.  Furthermore,  the  expected  output  of  cargo  turnover  and

- passenger  turnover  have  both  declined  to  a  certain  extent.  Meanwhile,  PEC

- continued   growing,   although   only  modestly,   with   its   improvement  unable  to

1.040

1.002

1.018

1.000

1.000

0.960

0.987

0.969

0.969

0.956

0.987

0.974

0.959

0.957

1.010

0.920

GML PEC BPC SCH

0.957

0.942

0.941

0.954

0.931

0.918

0.944

0.928

0.922

0.880

2012

2013

2014

Year

2015

2016

2017

Efficiency value

compensate for the decrease in BPC and SCH.

- Figure 3. GML and its decomposition during 2013-2017.

## 4.2.2 Regional level

- This research study also analyzed the dynamic efficiency of carbon emissions

- of transportation infrastructure and its decomposition at the overall national level.  In

- order to analyze the spatial distribution of carbon  emissions efficiency, we follow Bi

- et al. (2014) approach and group the 30 Chinese provinces by similar geographical

- location  and  level  of  economic  development.  That  resulted  in  three   regions of

- analysis, i.e., East, Middle and West. The partition in the three regions is listed in

Table 4.

Table 4. Three regions in China.

| East | Middle | West |
| --- | --- | --- |
| Liaoning, Beijing, Tianjin, Hebei, Shandong, Jiangsu, Shanghai, Zhejiang, Fujian, Guangdong, Hainan | Heilongjiang, Jilin, Shanxi, Henan, Hubei, Anhui, Hunan, Jiangxi | Inner Mongolia, Xinjiang, Ningxia, Qinghai, Gansu, Shaanxi, Sichuan, Chongqing, Guizhou, Yunnan, Guangxi |

- As shown in Figure 4, the GML of the three regions also had an approximate

- declining-rising-declining trend during the period 2001-2017. However, the GML

- values  among  the  three  regions  were  clearly imbalanced.  During  the observation

- period, the GML value  of the East  region was always  greater than  the Middle  and

- West regions. Similar to the results of Ren et al. (2018) and Bi et al. (2014), it can

- also be observed that the differences in regional economic development levels and

- the  level  of  optimization  of  industrial  structure  are  the  major  reasons  for  this

- inequality.

- As shown in Figure 5, there also were clear differences in the PEC (pure

- technical efficiency) values of the three regions (i.e. values for East > Middle >

- West regions). In general, PEC had an overall increasing trend in the second and

- third stages. However, in the first stage, the PEC of the East and Middle regions

- had an increasing trend, while the West region and the overall country experienced

- a downward  trend.  The  level  of R&D  resource  allocation  in the East   region  is

- significantly higher than in the central and western regions. This higher resource

- allocation provided a more favorable environment for technological progress in  the

- East region.

- As shown in Figure 6, the BPC of the three regions kept deceasing from 2001 to

- 2017. This indicates that the contemporaneous production frontier shifted towards

- the global production frontier (i.e., producing less desirable outputs and more CO2

- emissions).  This  also  means  that  reducing CO2  emissions  to  improve the current

- production frontier is a shared challenge for the three regions alike.

- As shown in Figure 7, the SCH of the three regions had a continuous downward

- trend.  This  indicates  that  the  scale  efficiency  of  all  regions  deteriorated  during

- 2001-2017. It also shows  that the optimization of the regional resources allocation is

- a pressing need to curb the decline of economies of scale in China. Yet, there are

- obvious differences between the three regions, as the East region’s SCH  deteriorated

- much more. This deterioration of the East economies may have been caused by the

excessive concentration of resources in this Chinese region.

Efficiency value

1.100

The first stage

The second stage

The third stage

1.050

1.000

0.950

0.900

Nation East region

Middle region

West region

0.850

2001 2002 2003 2004 2005 2006 2007 2008 2009 2010 2011 2012 2013 2014 2015 2016 2017

Year

Efficiency value

1.100

The first stage

The second stage

The third stage

1.050

1.000

0.950

0.900

0.850

0.800

Nation East region

Middle region

West region

0.750

2001 2002 2003 2004 2005 2006 2007 2008 2009 2010 2011 2012 2013 2014 2015 2016 2017

Year

Figure 4. GML of nation and three regions from 2001 to 2017.

- Figure 5. PEC of nation and three regions from 2001 to 2017.

1.050

The first stage

The second stage

The third stage

1.000

0.950

0.900

Nation

0.850	East Region Middle region

0.800

West Region

0.750

2001 2002 2003 2004 2005 2006 2007 2008 2009 2010 2011 2012 2013 2014 2015 2016 2017

Year

Efficiency value

- Figure 6. BPC of nation and three regions from 2001 to 2017.

1.100

The first stage

The second stage

The third stage

1.000

0.900

0.800

Nation

East region

Middle region West region

0.700

2001 2002 2003 2004 2005 2006 2007 2008 2009 2010 2011 2012 2013 2014 2015 2016 2017

Year

Efficiency value

- Figure 7. SCH of nation and three regions from 2001 to 2017

## 4.3 Analysis of the threshold effect

- According to the regression analyses above, environmental regulations (ER)

- have  both   positive  and   negative  effects  on   carbon   emissions  efficiency (CEE).

- However, ER may have a nonlinear effect on CEE when the threshold model is used

514	(Lu and Li, 2020).

- This research study uses the Tobit regression model to test the impact of ER on

- CEE before the threshold regression. Considering the existence of omitted variables

- and endogenous problems, the Tobit regression model with fixed effects and random

- effects was selected. Results are given in Appendix C of the Supplemental Material.

- Namely, Hansen’s panel threshold model was applied to analyze the threshold

- effect of environmental regulation (ER) on carbon emissions efficiency (CEE) for  the

- transportation  infrastructure.  Calculations  were  performed  with  StataMP  13.  As

- shown  in  Table  5,  only the test  for  the single  threshold  F  was  significant  with  a

- bootstrap p value of 0.003. Therefore, there is a single threshold effect of ER on CEE

- in the East region corresponding (0.540). This means there is a nonlinear effect of ER

on CEE in this region.

- Table 5. Test for threshold effects in the East region.

Threshold

Threshold

Critical values

| Region |  |  | F value | p value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | model | value |  |  | 1% | 5% | 10% |
|  | Single | 0.540 | 20.092*** | 0.003 | 15.158 | 8.156 | 5.895 |
| East region | Double | - | 1.952 | 0.220 | 8.186 | 4.483 | 3.297 |
|  | Triple | - | 2.382 | 0.113 | 6.784 | 4.273 | 2.527 |

- Note: *** means significant at 1%.

- As shown in Table 6, by comparing the significance levels, it is more likely to

- find a single or double threshold effect of ER on CEE in the Middle region. However,

- according to the maximum likelihood graph of the double threshold model in Figure

- 8, the second threshold value is difficult to identify. Hence, we consider that the

single threshold model is representative enough with a threshold value of 0.870.

- Table 6. Test for threshold effects in middle region.

Threshold

Threshold

Critical values

| Region | model | value | F value | p value | 1% | 5% | 10% |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Single | 0.870 | 18.825*** | 0.003 | 9.955 | 6.691 | 4.589 |
| Middle | Double | 2.060 | 3.238*** | 0.000 | 0.121 | -4.674 | -6.729 |
| region |  | 2.710 |  |  |  |  |  |
|  | Triple | - | 7.793 | 0.040 | 9.511 | 7.138 | 4.873 |

- Note: *** means significant at 1%.

- Figure 8. Maximum likelihood graph of the second threshold value in the double

threshold model for the Middle region.

- As shown in Table 7, only the test for the double threshold model was strongly

- significant with a bootstrap p value of 0.037 (threshold values of 0.820 and 0.890).

This means that there is a nonlinear effect of ER on CEE in the West region.

- Table 7. Test for threshold effects in the West region.

Threshold

Threshold

Critical value

| Region | model | value | F value | p value | 1% | 5% | 10% |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Single | - | 1.747 | 0.277 | 9.105 | 5.486 | 3.543 |
|  |  | 0.820** |  |  |  |  |  |
| West region | Double | 0.890** | 7.197** | 0.037 | 10.860 | 5.199 | 3.878 |
|  | Triple | - | 2.887 | 0.150 | 8.180 | 4.657 | 3.763 |

- Note: ** means significant at the 5% level.

- After the most suitable threshold models in each region were identified, they

- were applied to perform a regression analysis to estimate the ER’s coefficient. The

- regression results are shown in Table 8. The first is the East region. When ER is less

- than  0.540,  for  a  1%  increase  in  ER,  the  positive  impact  on  CEE  increases by

- 0.1810%, and the impact is significant. When ER is greater than 0.540, this positive

- effect increases by only 0.0065%, and the decrease rate is 96.2%. In the Middle

- region, the situation is similar to the East region. The difference is that, when ER in

- the Middle region is above the threshold value (ER = 0.870), the ER’s coefficient

- decreases  by 60.3% despite the significance level  remaining the  same.  As  for  the

- West region, when ER remains within the range of 0.820 to 0.890, the increase of ER

- can cause a significant negative effect on CEE. 561

- Table 8. Regression results of the corresponding threshold model in three

- regions.

Region	Threshold

Coefficient	Std.

p	95% confidence interval

|  | variable |  | Deviation | value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ER<0.540 | 0.181*** | 0.045 | 0.000 | 0.093 | 0.269 |
| East | ER≥0.540 | 0.006 | 0.01 | 0.500 | -0.012 | 0.025 |
| region | LnECO | -0.022 | .0162 | 0.183 | -0.054 | 0.01 |
|  | RES | -0.044 | 0.138 | 0.753 | -0.316 | 0.229 |
|  | OW | 0.009 | 0.087 | 0.922 | -0.162 | 0.179 |
|  | LnOPEN | 0.027*** | 0.01 | 0.008 | 0.007 | 0.048 |
|  | _cons | 0.845*** | 0.162 | 0.000 | 0.526 | 1.164 |
|  | ER<0.870 | 0.044*** | 0.01 | 0.000 | 0.024 | 0.063 |
|  | ER≥0.870 | 0.017*** | 0.005 | 0.001 | 0.007 | 0.028 |
| Middle | LnECO | 0.005 | 0.01 | 0.602 | -0.014 | 0.025 |
| region | RES | -0.4436*** | 0.127 | 0.001 | -0.694 | -0.192 |
|  | OW | -0.12*** | 0.043 | 0.007 | -0.021 | -0.034 |
|  | LnOPEN | -0.007 | 0.005 | 0.132 | -0.017 | 0.002 |

|  | _cons | 1.075*** | 0.08 | 0.000 | 0.917 | 1.233 |
| --- | --- | --- | --- | --- | --- | --- |
|  | ER<0.820 | 0.003 | 0.007 | 0.692 | -0.011 | 0.017 |
|  | 0.820≤ER<0.890 | -0.021*** | 0.008 | 0.009 | -0.036 | -0.005 |
|  | ER≥0.890 | -0.003 | 0.006 | 0.565 | -0.015 | 0.008 |
| West | LnECO | -0.003 | 0.005 | 0.483 | -0.013 | 0.006 |
| region | RES | -0.067 | 0.075 | 0.375 | -0.216 | 0.082 |
|  | OW | -0.03 | 0.026 | 0.261 | -0.082 | 0.022 |
|  | LnOPEN | 0.003 | 0.002 | 0.123 | -0.001 | 0.006 |
|  | _cons | 1.016*** | 0.046 | 0.000 | 0.926 | 1.107 |

- Note: *** means significant at 1%.

# 5. Discussion and policy implications

- At the national level, we have found that the CEE of transportation

- infrastructure had  a declining-rising-declining trend  during  2001  to  2017.  During

- 2001-2006,  the  declining  trend  was  mainly  caused  by the  low  level  of  the pure

- technical   efficiency.   Due   to   the   low   level   of   technology   innovation   in the

- transportation  industry,  the  low  utilization  rate  of  new  energy  was  unable  to

- effectively  promote  technology  progress  (Xie  et  al.,  2017).  In  the  period  of

- 2007-2012,  CEE  grew  mainly  because  of  the  improvement  in  pure  technical

- efficiency  and   the  contemporaneous  production  frontier.   In  2006,  the   Chinese

- government  issued  the  "China’s  National  Plan  to  address  Climate  Change" and

- thereafter gradually strengthened CO2 emissions control measures. The introduction

- of these policies had the affect of accelerating technology innovation across various

- industries, and thereby boosted economic development, as well as the use of new

- forms of energy, technologies, and materials (Chen et al., 2019). In summary, all

- these  actions  further  promoted  the  progress  of  technology  development  and

- innovation. However, in 2013-2017, CEE dropped sharply. This was mainly caused

- by deterioration of the contemporaneous production frontier and scale efficiency.

- At the regional level, there were obvious heterogeneities in the CEE of

- transportation  infrastructure  and  its  decomposition.  The  development  of  pure

- technical efficiency of the East region was always at a higher level than that of the

- Middle   and   West   regions.   From   a   regional   economic   perspective, industrial

- development was closely related to the regional economic development level (Zheng

- and  Wang,  2019a).  Hence,  due  to  the  obvious  differences  in  regional economic

- development levels across the three regions, it is reasonable to expect that the pure

- technical efficiency of the East region would have been relatively high. Meanwhile,

- compared with the East and Middle regions, the West region’s contemporaneous

- production frontier and scale efficiency evidenced a higher development trend. This

- was partially the consequence of the support received by the Western Development

- Plan of China. On implementing this plan, both transportation infrastructure and its

- energy efficiency improved in the West  region significantly (Zhang et  al., 2019).  It

- also improved its contemporaneous production frontier. In addition, China’s national

- level  “One  Belt  and  One  Road”  initiative  further  promoted  the  renovation  and

- construction of transportation infrastructure in the West region. This positive effect

- also relied on economies of scale in the West region (Liu and Xin, 2019) and reduced

- the deterioration of scale efficiency.

- The threshold regression results are a key focus area of this study. Overall, akin

- to the results of Chen et al. (2019) and Ren et al. (2018), environmental regulations

- seem to have  a nonlinear effect on  carbon  emissions  efficiency,  and  the  effects of

- environmental regulations are different in different regions. In this regard, Zhang et

- al. (2020) found  an inverted U-shaped relationship between  China’s  environmental

- regulations  and  carbon  emissions.  However,  our  study  has  further  identified the

- existence of some threshold effects of environmental regulations (ER) on carbon

- emissions efficiency. It has found that there is a single threshold in the East and

- Middle  regions,  whereas  there  are  double  thresholds  in  the  West  region.   More

- precisely, we find in the East region that when ER is >0.540, its coefficient becomes

- much smaller compared to ER<0.540. This means that ER in the East region should

- be controlled circa 0.540, a point at which it could really achieve the maximum

- improvement of CEE.

- At the same time, the control variable LnOPEN is significant and has a positive

- effect on CEE. This is contrary to the negative effect identified by Ding et al. (2018).

- Hence, it is shown that the so called "pollution paradise" hypothesis does not hold in

- the East region. This is due to the high level of economic development and strict

- environmental  regulations  in  the  East  region  that  effectively  supervise  foreign

- business activities. Namely, they cause some polluting foreign businesses to invest in

- other regions with loose ER, thereby improving the CEE of the East region.

- Similar results were observed in the Middle and West regions. Therefore, for the

- Middle  region,  a  balanced  relationship  between  investment  in  environmental

- pollution control with CEE needs to be explored. Contrary to the result of Ding et al.

- (2018), the technology innovation level has a significant negative impact on CEE.

- This  phenomenon  is  caused  by conflicting research  objectives. The Middle region

- has a large level of investment in scientific research, but due to the lack of an

- effective	scientific	and	technological	development	system,	the	scientific

- commercialization  rate  is  low.  In  addition,  state-owned  (OW)  assets  also  had  a

- significant negative impact on CEE. The excessive proportion of state-owned assets

- can  easily lead  to  monopoly problems  and  reduce  the  enthusiasm  for  using new

- technologies, new materials and new types of energy. Therefore, the Middle region

- can further support with social capital its participation in transportation infrastructure

- construction, operation and maintenance to alleviate this negative impact.

- Also contrary to the result of Ding et al. (2018), this research study shows that

- there may be an inverted "U" relationship between ER and CEE in the West region.

- When ER<0.820, CEE generally increases with the increase of ER, and when 0.820

- ≤ER, CEE decreases significantly with the increase of ER. This highlights that only

- a moderate level of environmental regulation can promote the progress of green

- technology. Moreover, when the government interferes excessively, environmental

- regulations becomes less  effective. Consequently,  the West  region should  keep ER

- below 0.820  as much  as  possible, and at  the same time, pay special attention to the

- possible negative impact on CEE when enhancing ER.

- As a result, in order to improve the CEE of transportation infrastructure, the

- following recommendations are provided for policy makers:

- (1) Encourage green technology innovation. This research study identified that

- technology progress is the main source of CEE growth for China's transportation

- infrastructure.  Therefore,  the  government  should  focus  on  policy  guidance  and

- stimulate  the  endogenous  power  of  companies  as  well  as  stimulating innovative

- practices.  For  example,  financial  subsidies  and  tax  incentives  can  be  granted to

- industrial	companies	undertaking	green	technology	innovation.	It	is	also

- recommended	to	create	channels	for	the	transformation	of	scientific	and

- technological achievements in green technology. These in addition to promoting the

- application  of  advanced  technologies  for  energy  conservation  and environmental

- protection   as   part   of   an   optimal   environment   for   technological  innovation.

- Furthermore, there is a need to enhance the level of human resources, technical

- competencies and training provision to underpin the overall innovation capacity of

- the Chinese transportation infrastructure. Finally, interregional technical  cooperation

- should  be  strengthened,   particularly  through   expanding  technical  support  from

- regions  with  a higher technology development  level  to  other  regions  with  lower

- levels.

- (2) Optimize the management approach to support CEE. This study identified

- that  the  deterioration  of  scale  efficiency was  the  major  reason  curbing  the CEE

- growth. Therefore, there is a need for more robust preliminary research to inform

- government  strategies  to  ensure  that  the  scale  of  transportation  infrastructure

- construction is compatible with local economic development plans. Adoption of an

- integrated systems (like big data) for transportation infrastructure should enhance the

- planning,  coordination  and linkage of  national  integrated transportation along with

- the optimization of resources allocation. Also, this study has found that the level of

- interaction in the East region with the outside world promotes the efficiency of basic

- transportation  carbon  emissions.  This  allows  achieving  a  win-win  situation  for

- transportation infrastructure development  and environmental protection.  Therefore,

- regarding the supervision of external capital, the Middle and West regions can learn

- from the East region’s experience.

- (3) Implement differential ER. This study identified that there are obvious

- regional differences regarding the impacts of ER on CEE. After 2013, ER in the East

- region had a significant negative impact. Therefore, the East region can reduce the

- intensity of ER. For the Middle region, the impact was positive and stable in the long

- term. Therefore, the Middle region can maintain the current level of ER intensity or

- even moderately increase ER to observe whether CEE improves. For the West region,

- the impact shifted between positive and negative, indicating the existence of an

- inverse U-shaped relationship. Hence, the West region should keep its ER intensity

- near its lowest value as much as possible.

- (4) Pay attention to the threshold effect of environmental regulations. Research

- shows that there is a threshold effect between ER and CEE.  In the East region, when

- environmental regulations are greater than the threshold, their role in promoting the

- carbon  emission  efficiency  of  transportation  infrastructure  drops  by  about 96%.

- Hence,  continuing to  increase the intensity of environmental  regulations  will likely

- cause a sharp increase in governance costs, while carbon emissions efficiency will

- not  increase  significantly.  Therefore,  the  East  region  should  carefully  adjust the

- intensity of environmental regulations in accordance with actual development needs

- for the region.  For the  Middle region,  when  environmental  regulations  are greater

- than the threshold, their promotion of CEE will drop to about 60%. At this point,

- continuing to  increase the intensity of environmental  regulations  can  play a certain

- role in promoting the growth of CEE. Therefore, when adjusting the intensity of ER

- in  the  Middle  region,   a  comprehensive  assessment  of  the  relationship  between

- governance costs and governance effects should be carried out to achieve a balance.

- Unlike the East and Middle regions, the intensity of ER in the West region will have

- a negative impact on CEE after reaching the threshold. Consequently, it can be

- observed  that  the  West  region  should  keep  environmental  regulations  below the

- threshold whenever possible.

# 6. Conclusions

- This research study evaluated the dynamic carbon emissions efficiency (CEE)

- of  China’s  transportation  infrastructure  during  the  2001-2017  period.  The  study

- applied   Hansen’s   panel   threshold   model   to   analyze  the  threshold   effects  of

- environmental regulations (ER) on CEE. The major findings are as follows:

- (1) During the observation period, the CEE of China's transportation

- infrastructure  had  a  declining-rising-declining  trend.  During  2001-2006, CEE

- decreased by approximately 4.9%, which was mainly caused by deterioration of

- the	production	frontier.	During	2007-2012,	CEE	increased	slowly	by

- approximately  1.43%,  which  was  mainly  caused  by  technology  progress and

- improvement  in  the  production  frontier.  During  2013-2017,  CEE  declined

- sharply by approximately 7.24%. This was mainly caused by the deterioration  of

- scale efficiency and the production frontier.

- (2) There are obvious regional heterogeneities in the CEE of transportation

- infrastructure and its decomposition between the three Chinese regions analyzed.

- The East region was obviously better than the Middle and West regions, while

- the  Middle  region  was  slightly  higher  than  the  West  region.  As  for  its

- decomposition, the pure  technical  efficiency of the East region was higher  than

- the other two regions, although its production frontier and scale efficiency was

lower than the West region in the same period.

- (3) ER had a threshold effect on CEE. For the East and Middle regions,

- there  was  a  single  threshold  effect,  with  thresholds  of  0.540  and  0.870

- respectively. For the West region, there was a double threshold effect (thresholds

- at 0.820 and 0.890). When ER in the East and Middle regions exceeded the

- threshold, its positive impact on CEE was reduced. However, when ER in the

- West region exceeded the threshold, its impact became negative.

- This study has improved the evaluation methods and theoretical frameworks for

- measuring the CEE of transportation infrastructure. This research has also provided

- evidence-based   recommendations  for   government  decision-makers  to  formulate

- improved ER policies. The major limitation of the study is that it did not consider

- different  types  of  regulation,  which  would  have  allowed  a  more  fine-grained

- discriminatory analysis. Hence, future research needs to be extended to explore how

different types of ER affect CEE.

# Acknowledgements

- This  research  is  supported  by  the  National  Natural  Science  Foundation  of China

- [grant  numbers  71942006];  National  Social  Science  Fund  Post-financing projects

- [grant numbers 19FJYB017]; List of Key Science and Technology Projects in China’s

- Transportation Industry in 2018-International Science and Technology Cooperation

- Project	[grant	numbers.2018-GH-006,	2019-MS5-100];	the	Natural	Science

- Foundation  of  Qinghai  Province  [grant  numbers  2020-ZJ-736].  The  fifth  author

- acknowledges the Spanish Ministry of Science and Innovation for his Ramon y Cajal

- contract (RYC-2017-22222) co-funded by the European Social Fund.

# Declaration of interests

- The authors declare that they have no known competing financial interests or personal

relationships that could have appeared to influence the work reported in this paper.

# Reference

Alpay, E., Kerkvliet, J. & Buccola, S. (2002). Productivity Growth and

- Environmental  Regulation  in  Mexican  and  U.S.   Food   Manufacturing. 752	American Journal of Agricultural Economics, 84(4), 887-901.

753		Ang, B. W. (1999). Is the Energy Intensity a Less Useful Indicator than 754	the Carbon Factor in the Study of Climate Change? Energy Policy, 27(15),  755	943-946.

756		Bai, J., Qu, J., Maraseni,  T.,  Wu,  J.,  Xu,  L.  &  Fan,  Y.  Spatial  and 757	Temporal Variations of Embodied Carbon Emissions in China’s Infrastructure. 758	Sustainability, 11(3), 749.

759		Bi, G.-B., Song, W., Zhou, P. & Liang, L. (2014). Does environmental 760	regulation affect energy efficiency in China's  thermal  power  generation?  761	Empirical evidence from a slacks-based  DEA  model.  Energy  Policy,  66, 762	537-546.

763		Brunnermeier,  S.  B.  &  Cohen,  M.  A.   (2003).   Determinants   of  764	environmental  innovation  in  US  manufacturing  industries.   Journal   of  765	Environmental Economics and Management, 45(2), 278-293.

766		Cao, H. & Li, H.  (2014). Simulation-based approach to modeling the  767	carbon   emissions   dynamic   characteristics   of   manufacturing   system  768	considering disturbances. Journal of Cleaner Production, 64, 572-580.

769		Chang, Y. T., Zhang, N., Danao, D. & Zhang, N. (2013). Environmental 770	Efficiency Analysis of Transportation System in China: A Non-Radial Dea 771	Approach. Energy Policy, 58, 277-283.

772		Chen,  X.,  Chen,  Y.  E.  &   Chang,   C.-P.   (2019).   The   effects   of 773	environmental regulation and industrial structure on carbon dioxide emission: 774	a non-linear investigation. Environmental Science and Pollution  Research, 775	26(29), 30252-30267.

776		Choi, Y., Zhang, N. & Zhou, P. (2012). Efficiency and Abatement Costs 777	of Energy-Related CO2 Emissions in  China:  A  Slacks-Based  Efficiency  778	Measure. Applied Energy, 98, 198-208.

- Chu, Z., Bian, S. & Li, B. (2014). POLICY EXTERNALITIES: HOW

- REFORM	AND	OPENING-UP	STRATEGY	AFFECTS	REGIONAL

- DISPARITY	OF	PRODUCTION	FACTORS	IN	CHINA.	Journal	of

- Investigative Medicine, 62(8), S66-S67.

- Cooper, W. W., Park, K. S. &  Pastor,  J.  T.  (2000).  RAM:  A  Range 784	Adjusted Measure of Efficiency. Journal of Productivity Analysis, 11, 5-42.

785		Ding, X., He, J. & Yu, X.  (2018).  Spatial  Differentiation  of  Carbon 786	Emission Efficiency of “Silk Road Economic Belt” based on Environmental 787	Regulation. IOP Conference Series: Earth and Environmental Science, 208, 788	012027.

789		Du, W., Wang, F. & Li, M. (2020). Effects of environmental regulation on 790	capacity utilization: Evidence from energy enterprises in China. Ecological 791	Indicators, 113, 106217.

792		Emrouznejad, A. & Yang, G.  L. (2016). CO2  Emissions Reduction of  793	Chinese  Light  Manufacturing  Industries:  A  Novel  RAM-Based  Global 794	Malmquist-Luenberger Productivity Index. Energy Policy, 96, 397-410.

795		Gómez-Calvet, R., Conesa, D., Gómez-Calvet, A. R. & Tortosa-Ausina, E. 796	(2014). Energy Efficiency in the European Union: What Can be Learned from 797	the Joint Application of Directional Distance  Functions  and  Slacks-Based 798	Measures? Applied Energy, 132(11), 137-154.

799		Gray, W. B. (1987). The  Cost  of  Regulation:  OSHA,  EPA  and  the  800	Productivity Slowdown. The American Economic Review, 77(5), 998-1006.

801		IPCC 2006.  2006  IPCC  Guidelines  for  National  Greenhouse  Gas  802	Inventories. Kanagawa: Institute for Global Environmental Strategies.

803		Jaffe, A.  B.  &  Palmer,  K.  (1997).  Environmental  Regulation  and  804	Innovation: A Panel Data Study. The Review of Economics and  Statistics,  805	79(4), 610-619.

- Ji,	Z.,	Li,	P.	&	Zheng,	X.	(2019).	MANUFACTURING

- AGGLOMERATION  AND  ENVIRONMENTAL EFFICIENCY IN CHINA:

- INSIGHTS FROM THE PANEL THRESHOLD MODEL. Transformations in

- Business & Economics, 18(1), 257-277.

- Lanjouw, J. O. & Mody, A. (1996). Innovation  and  the  International 811	Diffusion of Environmentally Responsive Technology. Research Policy, 25(4), 812	549-571.

813		Lanoie, P., Patry, M. & Lajeunesse, R. (2008). Environmental Regulation 814	and Productivity: Testing the Porter  Hypothesis.  Journal  of  Productivity  815	Analysis, 30(2), 121-128.

816		Liu, H. W., Zhang, Y., Zhu, Q. Y. & Chu, J. F.  (2017). Environmental  817	efficiency of land transportation in China: A parallel slack-based measure for 818	regional and temporal analysis. Journal of Cleaner Production, 142, 867-876. 819		Liu, Z. & Xin, L. (2019). Has China's Belt and Road Initiative Promoted 820	Its Green Total Factor Productivity? - Evidence from Primary Provinces along 821	the Route. Energy Policy, 129, 360-369.

822		Lu, J. & Li,  H.  (2020).  The  impact  of  government  environmental  823	information disclosure on enterprise location choices: Heterogeneity and

824	threshold effect test. Journal of Cleaner Production, 277, 124055.

825	Luo, Q. L., Miao, C. L., Sun, L. Y., Meng, X. N. & Duan, M. M. (2019).

826	Efficiency evaluation of green technology innovation  of  China's  strategic 827	emerging  industries:  An  empirical  analysis  based  on   Malmquist-data   828	envelopment analysis index. Journal of Cleaner Production, 238, 10.

829		Mielnik, O. & Goldemberg, J. (1999). Communication the Evolution of 830	the “Carbonization Index” in Developing Countries.  Energy Policy,  27(5), 831	307-308.

832		Oh, D.-h. (2010). A Global Malmquist-Luenberger Productivity Index. 833	Journal of Productivity Analysis, 34(3), 183-197.

834		Ouyang, X., Li, Q. & Du, K. (2020). How does environmental regulation 835	promote technological innovations in the industrial sector? Evidence from  836	Chinese provincial panel data. Energy Policy, 139, 111310.

837		Pan, X., Pan, X., Li, C., Song, J. & Zhang, J. (2019). Effects of China’s 838	environmental policy on carbon emission efficiency. International Journal of 839	Climate Change Strategies and Management, 11(3), 326-340.

840		Pei, Y.,  Zhu, Y.,  Liu, S., Wang,  X.  & Cao, J. (2019). Environmental  841	regulation and carbon emission: The mediation effect of technical efficiency. 842	Journal of Cleaner Production, 236, 117599.

843		Porter, M. E. (1991). America’s Green Strategy. Scientific American, 4, 844	168.

845	Quazi, A. & Talukder,  M. (2011). DEMOGRAPHIC  DETERMINANTS

846	OF    ADOPTION    OF    TECHNOLOGICAL    INNOVATION.    Journal of

847	Computer Information Systems, 52(1), 34-42.

848		Ren, S., Li, X., Yuan, B., Li, D. & Chen, X. (2018). The effects of three 849	types of environmental regulation on eco-efficiency: A cross-region analysis in 850	China. Journal of Cleaner Production, 173, 245-255.

851		Ren, X., Hu, Z. & Xiang, Y.  (2011).  Reasonable  analysis  on  spatial 852	distribution of transportation infrastructure and allocation of transportation 853	investment in China - from the Perspective of Manufacturing Location Choice. 854	Proceedings of  the  2011  International  Conference  on  Remote  Sensing, 855	Environment and Transportation Engineering (RSETE 2011), 573-576.

856		Schou, P. (2002). When Environmental Policy Is Superfluous: Growth 857	and Polluting Resources. Scandinavian Journal of Economics, 104(4).

858		Shen, Q., Sun, T. & Luo, K. (2019). Research ON China’s tourism carbon 859	emission efficiency and its influencing  factors.  Fresenius  Environmental  860	Bulletin, 28, 6380-6388.

861		Sheng, P., Li, J., Zhai, M. & Huang, S. (2020). Coupling of economic  862	growth and reduction in carbon emissions at the efficiency level: Evidence  863	from China. Energy, 213, 118747.

864		Sinn, H.-W. (2008). Public Policies against Global Warming: A Supply 865	Side Approach. International Tax and Public Finance, 15(4), 360-394.

866		Sueyoshi, T., Goto, M. & Ueno, T. (2010). Performance Analysis of US 867	Coal-Fired Power Plants by Measuring Three DEA Efficiencies. Energy Policy, 868	38(4), 1675-1688.

869		Wang, Q. & Wang, S. (2020). Preventing carbon  emission  retaliatory 870	rebound post-COVID-19 requires expanding free trade and improving energy 871	efficiency. Science of The Total Environment, 746, 141158.

872		Wang, Q. & Yuan, Q. (2020). Energy-saving  and  emission  reduction 873	potential of the tobacco  industry:  A  case  study  of  China's  18  cigarette  874	enterprises. Journal of Cleaner Production, 244, 13.

875		Wang, W. W., Zhang, M. & Zhou, M. (2011). Using LMDI Method to  876	Analyze  Transport  Sector  CO2  Emissions   in   China.   Energy,  36(10), 877	5909-5915.

878		Wang, Y., Zuo, Y., Li, W.,  Kang, Y.,  Chen, W.,  Zhao, M. & Chen, H.  879	(2019). Does environmental regulation affect CO2 emissions? Analysis based 880	on threshold effect model. Clean Technologies and  Environmental  Policy, 881	21(3), 565-577.

882		Wang, Z. & He, W. (2017). CO2 Emissions  Efficiency  and  Marginal 883	Abatement  Costs  of  the  Regional   Transportation   Sectors   in   China.   884	Transportation Research Part D-Transport and Environment, 50, 83-97.

885	WRI 2017. Climate Data Explorer.

886		Xie, R.,  Fang,  J.  &  Liu,  C.  (2017).  The  effects  of  transportation  887	infrastructure on urban carbon emissions. Applied Energy, 196, 199-207.

888		Zhai, X. & An,  Y.  (2020).  Analyzing  influencing  factors  of  green  889	transformation  in  China’s  manufacturing  industry  under   environmental 890	regulation: A structural equation model. Journal of Cleaner Production, 251, 891	119760.

892		Zhang, C., Zhou, B. & Wang, Q.  (2019).  Effect  of  China's  Western 893	Development Strategy on Carbon Intensity. Journal of Cleaner  Production, 894	215, 1170-1179.

895		Zhang, J., Li, H., Xia, B. & Skitmore, M. (2018). Impact of Environment 896	Regulation on the Efficiency of Regional Construction Industry: A 3-Stage 897	Data Envelopment Analysis (DEA). Journal of  Cleaner  Production(200),  898	770-780.

899		Zhang, W., Li, G., Uddin, M. K. & Guo, S. (2020). Environmental 900	regulation, Foreign investment behavior, and carbon emissions for 30

901	provinces in China. Journal of Cleaner Production, 248, 119208.

902		Zhao, X., Yin, H. & Zhao, Y. (2015). Impact of environmental regulations 903	on the efficiency and CO2 emissions of power plants in China. Applied Energy, 904	149, 238-247.

905		Zheng, H.-H. & Wang, Z.-X. (2019a). Measurement and comparison of 906	export sophistication of the  new  energy  industry  in  30  countries  during 907	2000-2015. Renewable & Sustainable Energy Reviews, 108, 140-158.

| 908 | Zheng, H. & Wang, Z. (2019b). Measurement and Comparison of Export |
| --- | --- |
| 909 | Sophistication of the New Energy Industry in 30 Countries during 2000-2015. |
| 910 | Renewable and Sustainable Energy Reviews, 108, 140-158. |
| 911 | Zhou, N., Price, L., Dai, Y., Creyts, J., Khanna, N., Fridley, D., Lu, H., |
| 912 | Feng, W., Liu, X., Hasanbeigi, A., Tian, Z., Yang, H., Bai, Q., Zhu, Y., Xiong, |
| 913 | H., Zhang, J., Chrisman, K., Agenbroad, J., Ke, Y., McIntosh, R., Mullaney, D., |
| 914 | Stranger, C., Wanless, E., Wetzel, D., Yee, C. & Franconi, E. (2019a). A |
| 915 | Roadmap for China to Peak Carbon Dioxide Emissions and Achieve a 20% |
| 916 | Share of Non-Fossil Fuels in Primary Energy by 2030. Applied Energy, 239, |
| 917 | 793-819. |
| 918 | Zhou, P., Ang, B. W. & Han, J. Y. (2010). Total factor carbon emission |
| 919 | performance: A Malmquist index analysis. Energy Economics, 32(1), 194-201. |
| 920 | Zhou, Y., Liu, W., Lv, X., Chen, X. & Shen, M. (2019b). Investigating |
| 921 | interior driving factors and cross-industrial linkages of carbon emission |
| 922 | efficiency in China's construction industry: Based on Super-SBM DEA and |
| 923 | GVAR model. Journal of Cleaner Production, 241, 118322. |
| 924 | Zhou, Y., Xu, Y., Liu, C., Fang, Z., Fu, X. & He, M. (2019c). The |
| 925 | Threshold Effect of China’s Financial Development on Green Total Factor |
| 926 | Productivity. Sustainability, 11(14). |
| 927 | Zofı́o, J. L. & Prieto, A. M. (2001). Environmental Efficiency and |
| 928 | Regulatory Standards: The Case of CO2 Emissions from OECD Industries. |
| 929 | Resource and Energy Economics, 23(1), 63-83. |
| 930 |  |

Highlight：

## Impact of Environmental Regulations on Carbon Emissions of Transportation Infrastructure: China’s Evidence

- Using Chinese transportation infrastructure 2001-2017 panel data as an empirical study.

- Carbon emission efficiency experienced a 10.15% decrease during 2001-2017.

- The difference in carbon emission efficiency among regions is significant.

- There is a non-linear relationship between ER and carbon emission efficiency.

Declaration of interests

☒ The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

- The authors declare the following financial interests/personal relationships which may be considered as potential competing interests: