Classification of Cortical Bone Thicknesses Based on RF Signal Spectral Analysis

Hossam H. Sultan1, Enrico Grisan1, Laura Peralta2, Sevan Harput1

1South Bank Applied Bioengineering Research, London South Bank University, London, UK

2School of Biomedical Engineering & Imaging Sciences, King’s College London, London, UK

E-mail: abdelh13@lsbu.ac.uk

Abstract— Cortical bone thickness is an important biomarker of bone fragility that reveals the risk of fractures. However, ultrasound bone assessment is challenging due to the complex nature of bone, such as varying porosity and microarchitecture, and the large difference between bone and soft tissue acoustic impedances.

The research objective of this study is to develop a method to estimate cortical bone thickness by using spectral analysis, while avoiding traditional speed of sound measurements’ figures due to the porous structure of bone tissue. In this study, multi-frequency ultrasound acquisitions have been used to cover a wide range of bone thickness and porosity values. Frequency modulated chirp waveforms are used as a transmit signal to increase the measurement SNR and the continuous wavelet transformation (CWT) is employed for the spectral analysis. The feasibility of the proposed methodology is demonstrated on simulated datasets and via experiments using ex vivo bone tissue. The preliminary experimental results showed a potential for cortical thickness classification using the received RF data.

Keywords—Bone characterization, Continuous wavelet transformation, Deep learning, Chirp signal.

# Introduction

Human bone is a mineralized and rigid organ that comprises the main part of the skeleton. It acts as the main part of the human skeleton. It has different vital functions including red and white blood cells production, and minerals storage [1]. Cortical bone thickness and porosity is a considerable biomarker of bone fragility that reveals the risk of fractures [2, 3]. Hence, there is a clinical need to assess cortical bone microstructure quality in order to reduce risk of fracture.

During the last few decades, many ultrasound-based approaches have been introduced to assess bone micro and macro structures. The very early beginning of employing ultrasound technology in the field of bone assessment has been started in 1990’s [1]. Then, different Quantitative ultrasound (QUS) based technique has been introduced. Some QUS methods depend on ultrasound attenuation during passing through living bone. However, it is limited to peripheral sites only which is affect its reliability to scan and assess different bone sites. In [2], an ex-vivo study has been introduced to compare between ultrasound and Micro-CT for both porosity and thickness measurements. Nowadays, most common approaches use synthetic aperture and compound imaging to produce a relatively high-resolution scan. This process is computationally expensive and consumes much time during processing.

Scalograms is a visual representation of the continuous wavelet transform (CWT) absolute coefficients values of the input data. CWT is also sensitive to the damping ratio of oscillating signals [3] and Transient features. In addition, the use of coded chirp signals increases the (SNR) for a longer distance operation. Up-chirp signals with tukey window and 60% fractional bandwidth were used for excitation. Deep learning approaches, e.g. Convolutional Neural Network (CNN), has the ability to automatically extract features and overcome the former problems, in which higher levels identify more complex information by aggregating lower levels features. CNNs is extensively used in image analysis and is designed to require minimal pre-processing.

In this study, a cortical bone classification based on RF data spectral analysis and deep neural networks is introduced. It comprises 3 major phases. The first phase is the simulation and transformation of the RF signal to wavelet visual representation. The second phase involves features extraction and data summarization using different deep neural networks (AlexNet, Googlenet, Inceptionv3, Resnet18, and VGG16), while the third phase includes a real animal bone scanning based on single pulse-echo acquisitions. To the extent of our knowledge, this is the first study that quantify cortical bone thickness using the spectral data coming from the received RF data.

# Methods

To verify the influence of cortical bone thickness on the visual wavelet representation of the RF signal, different combinations of cortical thicknesses, porosities, and frequencies were simulated. Besides, a real scan validation has been introduced to validate the proposed methodology.

## Simulation setup

The simulation setup is shown in Figure 1. A simulation-based on a 2D finite-difference time-domain method (Simsonic) [4] was used for cortical bone simulation with varying porosity (0-20%) and thickness levels (1-8 mm) [4]. The cortical bone was modeled as a linear elastic 2D plate, and various structures were established to mimic intact and pathological tissues. It is assumed that the cortical plate is totally immersed in soft tissue-like medium and the model is surrounded by perfectly matched layers (PML) to limit high reflections from the boundaries.

Equation 1 describes the elastodynamic propagation of mechanical waves in continuous media obeying Hooke's law (Equation 2) [1].

ρx∂vi∂tx,t= j=1d∂Tij∂xjx,t+ fix,t                        1

∂Tij∂tx,t= j=1di=1dcijklx ∂vk∂xlx,t+ θijx,t         2

,where, x and t are the space and time variables, ρx is mass density, c(x) is the fourth order rigidity tensor. v(x,t) are the vector components of the particle velocities. T(x,t) are the components of the stress tensor. fi and θi are source terms, in which f is the vector components of force sources and θ is the tensor components of strain sources.

A typical 128 elements linear transducer array with multiple centre frequencies was used. The transducer is immersed in soft tissue-like medium and used as a transmit/receive array which is placed 1 mm below the soft tissue surface and parallel to bone surfaces.

Figure 1: Simulation setup aligned with the cortical bone surface.

## Chirp signal and scalograms

Chirp signals with multiple centre frequencies (1-8 MHz) were used as excitation pulses. They provide a constant source of acoustic energy, increase the efficiency of converting electric energy to acoustic energy, and increases the SNR for a longer distance operation. Up-chirp signals with tukey window were used. Figure 2 (Left) shows an up-chirp signal with tukey window with a centre frequency of 5 MHz and bandwidth of 2 MHz. Based on the ultrasound beam profile, central 20 echoes were acquired from each simulation. The absolute value of the continuous wavelet transform of the signals has been calculated to transform the 1D received echo to a 2D visual representation (scalograms) afterwards. Scalograms include time, scale, and coefficient value as in Figure 2 (Right).

By definition, CWT is a convolutional operation of the input data with a set of other functions generated by the mother wavelet to measure the similarity between the input signal and the analyzing function. The time signal is multiplied by a shift version of the wavelet. Then, absolute wavelet coefficients as a function of scale and position are given as the output as follows:

Ta,b=1a -∞∞xtψ*t-badt                              3

, where ψ*(t) is the complex conjugate of the mother wavelet, and a and b are scale and position respectively [5].

Figure 2: (Left) Transmitted chirp signal with a centre frequency of 5MHz and bandwidth of 2MHz, (Right) the 2D representation of the signal showing the absolute coefficient values of the CWT.

The scalogram representation for a simulated 4mm bone sample at 5MHz is shown in Figure 3. The cone of influence represents areas potentially affected by edge artifacts. These effects grow from regions where the stretched wavelets extend behind the edges of the observation interval. They can suppress the DC components in the received signal, and the corresponding spectral distribution could be considered to estimate the thickness inside the targeted medium.

Figure 3: (Left) A reflected 5MHz chirp signal, (Right) represents the scalogram response that shows boundaries’ reflections coming from the same regions in the time-domain.

## Convolutional neural networks

Figure 4 shows the proposed pipeline, each received channel is processed separately. Then, scalograms have been fed to a CNN to hierarchy extract the most describing features in the image. CNN use multinomial logistic regression to normalize the scalograms to a probability distribution over pre-outlined labels, and able to translate the scalograms without additional processing or speed-of-sound calculations. A softmax layer is used as a normalized exponential function

Accuracy metrics Training Data Partitioning Network Training Training Accuracy

Raw RF data

Scalograms

representation

Figure 4: Different topologies of convolutional neural networks are trained with the spectral transformations which are derived from the received RF-data.

to squeeze all of the predicted classes into a defined range between (0 and 1) as in equation (1).

y zj =ezjk=1kezk                                                            4

, where the probability of any class (j) can be calculated over (k) different classes as a function y (z), and their total values are equal to 1.

To estimate the training loss and provide the final categorical label for each input, a classification layer based on cross entropy loss is used. Loss is calculated as follows:

H p, q=- x px* log q x                    (5)

, where p is the target label vector, and q (x) is the output vector from the normalized exponential function (softmax layer).

The proposed models are trained on Intel i7-7700HQ CPU @ 2.80GHz, NVIDIA GeForce GTX 1060 (6 GB) GPU, 16.0 GB RAM. All models are validated using the simulated data (6,400 data points). The data have been partitioned into 20% and 80% for testing and training respectively. 20 epochs have been trained with a normal distribution to set the initial layers weights, initial learning rate of 0.001, mini-batch size of 16, and SGDM optimizer to update the training weights. This was followed by the training stage and performance computations.

# Results and discussion

## Scalograms

Figure 5 shows an example of a received RF signal of the received signal, that reveals the proximal and distal interfaces of the simulated bone, from a single channel and its converted scalogram. The high-intensity (bright yellow) area manifests the strong reflection from the first soft tissue-bone interface. In contrast, the low intensity marked area shows the reflection from the distal boundary.

## Single frequency vs. multi-frequency response

When all networks are compared, Alexnet showed a good compromise between the overall accuracy and training speed [6]. Inception_v3 model has achieved the best average overall accuracy of 73.44 %. However, it consumed around 53 hours for training due to its complex structure (315 layers, and depth of 48). Alexnet has an average single frequency accuracy of 70.31% achieved at 5 times lower training time compared to Densenet. 8 MHz showed the lowest accuracy among all frequencies; however, it showed a good performance for low thickness values (1 and 2 mm). As the wavelength is a function of frequency and speed-of-sound, a multi-frequency approach has been applied to ensure greater consistency. Alexnet showed an overall accuracy of 99% after combining the class labels from all applied frequencies.

Figure 5: (Top) Received signal from element(66th) for 4mm thickness, 5% porosity, and 5 MHz (Bottom) CWT shows the reflection from both proximal and distal bone interfaces.

## GradCAM

Figure 6 (right) shows that the Gradient Class Activation Map (GradCAM) for the given scalogram in Figure 6 (left) successfully focuses on the most discriminative image regions for classification. It uses the gradients of the target rolling into the final convolutional block to generate a coarse localization map highlighting the most important regions in the image used for classification.

Figure 6: (Left) the scalogram of the received RF signal (5mm ct. th., 15% porosity, and 5MHz), (Right) GradCAM for the same image.

## Ex-vivo validation using a single element transducer

Experiment setup of the validation part is shown in Figure 7. An animal bone sample (ovine) is used to validate the proposed methodology. A single element transducer with a centre frequency of 5 MHz, operating from 2.89 MHz to 6.49 MHz, is used. A multiple frequency ranges have been tested with a bandwidth of 2 MHz within the transducer bandwidth. Both transducer surface and bone sample are immersed in degassed water, and the transducer surface is aligned parallel to the cortical bone surface. The total dimensions of the sample is 25 mm, 27 mm, and 23 mm in axial, lateral, and radial directions. The cortical bone thickness is around 1.3 mm. Figure 8 shows the produced scalograms for frequencies 3, 4, and 5 MHz. All these responses have been classified as 1mm. However, the preliminary results should be further investigated, as only a single animal bone sample has been scanned to validate the proposed method, due to time limitations.

Figure 7: Experiment setup using a single element transducer.

Figure 8: Scalograms for the reflected RF data at frequencies 3, 4, and 5 MHz from left to right.

# Conclusion

This work presented an automated numerical study aiming to classify cortical bone thicknesses based on the spectral analysis of ultrasound wave propagation in healthy and osteoporotic bones. Multi-frequency pulse-echo approach enabled to acquire large dataset (a total number of 6,400 RF data points) which is a combination of 8 different thicknesses, 5 different porosities, and 8 frequencies. It is demonstrated that CNN can be used for extracting the transient features from the continuous wavelet transformations of the received RF data. The outcomes showed the potential of CNNs and multi-frequency acquisition to be used in cortical bone thickness quantification.

##### References

[1]  P. Laugier, B. Fournier, and G. Berger, "Ultrasound parametric imaging of the calcaneus: in vivo results with a new device," Calcified tissue international, vol. 58, no. 5, pp. 326-331, 1996.

[2]  J.-G. Minonzio et al., "Bone cortical thickness and porosity assessment using ultrasound guided waves: An ex vivo validation study," Bone, vol. 116, pp. 111-119, 2018.

[3]  J. Slavič, I. Simonovski, and M. Boltežar, "Damping identification using a continuous wavelet transform: application to real data," Journal of Sound Vibration, vol. 262, no. 2, pp. 291-307, 2003.

[4]  E. Bossy, M. Talmant, and P. Laugier, "Three-dimensional simulations of ultrasonic axial transmission velocity measurement on cortical bone models," The Journal of the Acoustical Society of America, vol. 115, no. 5, pp. 2314-2324, 2004.

[5]  P. Addison, "Introduction to redundancy rules: the continuous wavelet transform comes of age,"  vol. 376, ed: The Royal Society Publishing, 2018, p. 20170258.

[6]  H. Sultan, E. Grisan, L. Peralta, and S. Harput, "Estimation of Cortical Bone Strength Using CNN-based Regression Model," presented at the IEEE International Ultrasonics Symposium, 2022.